Processor Design Studio

A Unified Platform for Processor Simulation, Architecture Exploration, and Comparative Performance Analysis

Review-1 Submission Document

Prepared by: Bikarm Sardar

Registration Number: 26MAI0065

Domain: Computer Architecture, Microarchitecture Simulation, Parallel Computing

# 1. Substantiation of Problem with Justification

## 1.1 Background

With the physical decay of Dennard Scaling and the slowdown of Moore's Law, processor performance gains can no longer be achieved simply by scaling clock frequencies or shrinking transistor dimensions; gains increasingly require domain-specific architectures, custom instruction extensions, and intricate memory-hierarchy optimization. Evaluating such changes via physical silicon tape-outs or HDL (Verilog/VHDL) is expensive and slow:

- Prototyping cost: fabricating a modern IC prototype at advanced nodes (sub-7nm) runs into the tens of millions of dollars per iteration, with total design costs at leading-edge nodes reported well above $100M by industry cost analyses (IBS, IDC).

- Re-spin risk: a substantial share of commercial SoC revisions require costly hardware re-spins due to microarchitectural bottlenecks — pipeline stalls, cache thrashing, branch misprediction — undetected before synthesis.

- Observability: physical silicon and FPGA prototypes provide minimal visibility into pipeline latches, hazard conditions, or cycle-by-cycle cache behaviour.

## 1.2 Why Now, and Why Software

- Open ISAs (RISC-V) and heterogeneous/multi-core workloads increase demand for accessible, parallel-capable simulation environments over slow full-system simulators (e.g. gem5, which runs orders of magnitude slower than physical execution).

- Software simulation gives full observability (every latch, register, cache line, every cycle), instant reconfigurability (no re-synthesis), and parallel exploration of many configurations at once via OpenMP.

## 1.3 Primary Use Cases

- Design-space exploration: sweeping cache size/associativity/replacement policy and branch-predictor choice to study power-performance-area trade-offs before RTL is written.

- Pedagogy: letting students trace instructions through the 5-stage pipeline cycle by cycle, observing stalling versus forwarding.

- Compiler/firmware study: evaluating assembly-level optimizations against RAW hazards and control-branch penalties.

# 2. Survey — Comparative Analysis of Existing Solutions

## 2.1 Academic Publications

| # | Title | Authors, Year, Venue | Access | Gap Identified |
| --- | --- | --- | --- | --- |
| 1 | Survey of CPU and Memory Simulators in Computer Architecture | Hwang et al., 2025, Simulation Modelling Practice and Theory (Elsevier), DOI 10.1016/j.simpat.2024.103032 | Licensed (RightsLink #6336060626802) | Full-system simulators have steep setup complexity, no real-time interactive UI |
| 2 | Simulators for Processors Used in Virtualization: A Survey | Mhatre & Chandran, 2025, IEEE Open Journal of the Computer Society, DOI 10.1109/OJCS.2025.3591359 | Open Access | No native parallel-execution framework for multi-parameter sweeps |
| 3 | WebRISC-V: A 32/64-bit RISC-V pipeline simulation tool | Mariotti & Giorgi, 2022, SoftwareX (Elsevier), DOI 10.1016/j.softx.2022.101105 | Open Access (CC BY) | Web pipeline visualiser only — one config at a time, no cache/predictor sweep, no OpenMP DSE |
| 4 | SonicRV: An Educational Platform for Web-Based Simulation and Visualization of RISC-V Processor Architectures | Große, Reitinger & Klemmer, 2026, IEEE Access, DOI 10.1109/ACCESS.2026.3656022 | Open Access | Single/multicycle/pipeline visualiser; no architecture-parameter sweep or parallel exploration |
| 5 | FREESS: An Educational Simulator of a RISC-V-Inspired Superscalar Processor Based on Tomasulo's Algorithm | Giorgi, 2025, WCAE'25 (ACM), DOI 10.1145/3743646.3750018 / arXiv:2506.07665 | Open Access | Superscalar/Tomasulo visualiser for one configuration; no cache/branch-predictor sweep or parallel DSE |
| 6 | A design methodology for soft-core platforms on FPGA with SMP Linux, OpenMP support, and distributed hardware profiling system | Muttillo et al., 2016, EURASIP Journal on Embedded Systems, DOI 10.1186/s13639-016-0051-9 | Open Access (CC BY 4.0) | OpenMP-based DSE, but for FPGA soft-core hardware synthesis — not an ISA-level pipeline/cache/predictor simulator with a web UI |

## 2.2 Patent Filings

| Patent | Title | Assignee | Gap Identified |
| --- | --- | --- | --- |
| US 2016/0063164 A1 | Method and System for Hardware/Software Co-simulation and Processor Resource Allocation | — | Host-level scheduling only, no microarchitectural component-level observability |
| US 7,788,078 B1 | Processor/Memory Co-Exploration at Multiple Abstraction Levels | CoWare Inc. / Synopsys Inc. | Proprietary ADL toolchain; no interactive visualisation or parallel sweep |
| US 9,928,179 B2 | Cache Replacement Policy | Intel Corporation | Hardware replacement-policy mechanism; not a configurable simulation/exploration tool |
| US 6,263,427 B1 | Branch Prediction Mechanism | Rise Technology Co. | Hardware branch-target-buffer design; not simulated or comparatively configurable |

## 2.3 Existing Software Products / Tools

gem5:

Modular full-system research platform. Cons — high complexity, steep learning curve, CLI-only, no interactive pipeline playback.

SimpleScalar / MARSSx86:

Legacy academic toolsets. Cons — largely unmaintained, no multi-core parallel sweep capability, no modern web UI.

WebRISC-V / SonicRV / FREESS (tool form of Publications 3–5 above):

Strong cycle-by-cycle pipeline or superscalar visualisation in the browser. Cons — each evaluates one architecture configuration at a time; none sweep across cache, associativity, or branch-predictor parameters, and none parallelise that sweep.

# 3. Comparative Analysis & Gap Identification

| Reference | Key Conclusion / Gap |
| --- | --- |
| Hwang et al. (2025) | Existing tools trade simulation speed against microarchitectural accuracy; usability is often secondary. |
| Mhatre & Chandran (2025) | Tools lack native parallel frameworks for multi-parameter sweeps. |
| WebRISC-V / SonicRV / FREESS | Excellent single-configuration visualisation; no parameter sweep, no parallel exploration. |
| Muttillo et al. (2016) | OpenMP-based DSE proven effective, but for FPGA hardware synthesis, not ISA-level software simulation. |
| US 7,788,078 B1 / US 9,928,179 B2 / US 6,263,427 B1 | Real ADL, cache, and branch-predictor mechanisms, but fixed hardware designs, not comparative exploration tools. |
| gem5 | Industry-accurate but heavy, complex, and not visually interactive. |
| Proposed: Processor Design Studio | Combines cycle-accurate simulation, parallel OpenMP sweeps, and web visualisation. |

| Feature | Lit. / Patents | Educational Tools* | gem5 | Proposed Studio |
| --- | --- | --- | --- | --- |
| 5-stage cycle-accurate pipeline | Partial | Yes | Yes | Planned |
| RAW hazard detection & forwarding | Conceptual | Yes | Yes | Planned, toggleable |
| Configurable cache (LRU/FIFO/Rand) | Yes | No | Yes | Planned |
| Dynamic branch prediction | Yes | Partial | Yes | Planned, 4 types |
| Parameter sweep across configs | No | No | Manual only | Planned |
| OpenMP-parallelised sweep | No / thread-alloc only | No | No (manual MPI) | Planned, native OpenMP |
| Interactive web UI | No | Yes | No | Planned, React |
| Open-source, zero cost | Mostly proprietary | Yes | Open-source | Planned, open-source |

*WebRISC-V, SonicRV, FREESS.

## 3.1 Identified Gap and Proposed Contribution

No single publication, patent, or tool in this survey combines all four of: (a) a cycle-accurate pipeline with toggleable hazard detection and forwarding, (b) a configurable cache hierarchy across multiple replacement policies, (c) multiple branch-predictor models, and (d) an OpenMP-parallelised sweep across those configurations surfaced through an interactive web dashboard. The educational visualisers (WebRISC-V, SonicRV, FREESS) are strong at (a)–(c) for one configuration at a time but do not sweep or parallelise across the design space. Research and vendor tools (gem5; the CoWare/Synopsys ADL flow) support configuration exploration but are heavyweight, CLI-driven, or proprietary, with no comparison dashboard. The one directly comparable OpenMP-driven design-space-exploration methodology found (Muttillo et al., 2016) applies the same idea — simulate, sweep, parallelise — but to FPGA soft-core hardware synthesis, not to ISA-level software simulation of pipeline/cache/predictor trade-offs.

This is the gap Processor Design Studio is proposed to fill. The new contribution is the combination itself, not any single component in isolation: an OpenMP-parallelised sweep runner (Section 4.2) driving a cycle-accurate simulation core, with results surfaced as directly comparable charts in a web dashboard — turning a manual, one-configuration-at-a-time exercise (as in the closest existing tools) into a parallel, multi-configuration exploration workflow.

# 4. Proposed Methodology

Three decoupled layers are proposed: a C++17/OpenMP simulation core, a Spring Boot backend invoking it as a subprocess, and a React frontend for configuration and visualisation. The core requires OpenMP (hence C/C++); the backend and UI favour faster iteration in Java and JavaScript.

[ Assembly (.asm) ] -> [ Two-Pass Assembler ] -> [ 32-bit Machine Code ]

[ Config (JSON) ] -> [ 5-Stage Pipelined Sim ] -> Cache | Branch Predictor | OpenMP Sweep

[ Visual Dashboard ] <- [ REST API ] <- [ SimResult JSON Output ]

## 4.1 Planned Algorithms

- Two-pass assembler: pass 1 builds a label-to-address table; pass 2 emits 32-bit R-type/I-type/J-type encoded instructions.

- Latch-snapshot pipeline: each stage reads a snapshot latch and writes a next-cycle latch, avoiding intra-cycle read-after-write anomalies.

- Hazard/forwarding: load-use stall triggers when an ID/EX load's destination matches an IF/ID source register; EX/MEM->EX and MEM/WB->EX bypass paths planned, priority to the younger producer.

- Cache: standard set-index/tag computation; planned LRU, FIFO, and Random eviction policies.

- Branch prediction: static baselines plus a per-PC 2-bit saturating counter; 2-stage flush on misprediction.

- OpenMP sweep: independent configs distributed via #pragma omp parallel for schedule(dynamic,1) — no shared mutable state across threads.

# 5. Planned Experimental Setup

- Compiler: GCC/MinGW-w64 (C++17), -O2 -fopenmp -Wall -Wextra  |  Build: CMake  |  Backend: Java 17 / Spring Boot / Maven  |  Frontend: React 18 / Vite / Recharts

| Benchmark | Workload | Instruction Mix |
| --- | --- | --- |
| loop_sum.asm | Tight counter loop | ADDI, ADD, BEQ, J, HALT |
| factorial.asm | Recursive calls | JAL, JR, LW, SW, ADDI, SUB |
| bubble_sort.asm | In-place sort (10 elems) | LW, SW, BLT, BGE, ADDI, ADD |
| array_sum.asm | Linear memory traversal | LW, ADD, ADDI, BNE |
| hazard_heavy.asm | Dense RAW/load-use hazards | Dependent ADD, LW, SUB |
| matrix_multiply.asm | 3x3 matrix multiply | Nested loops; LW, SW, SLL, BEQ |

# 6. Expected Outcomes & Evaluation Plan

Implementation follows this review, so no empirical results are reported here. The table below fixes what will be measured and the hypothesis each measurement tests, so results can be reported against this plan in the next review.

| Category | What Will Be Measured | Hypothesis to Test |
| --- | --- | --- |
| Pipeline | Cycles, CPI, IPC, stalls per benchmark/config | Forwarding reduces CPI vs. stall-only on hazard-dense code |
| Cache | Hit rate, misses per size/associativity/policy | Higher associativity + LRU improves hit rate, with diminishing returns |
| Branch prediction | Misprediction rate per predictor type | 2-bit saturating matches/beats static on loops, wins on irregular branches |
| OpenMP scaling | Wall time, speedup, efficiency at 1/2/4/8 threads | Serial fraction (I/O, aggregation) bounds speedup per Amdahl's Law |

Results will be presented as comparison tables and charts generated by the React dashboard, included in the subsequent review once implementation is complete.

# 7. Anticipated Limitations & Future Work

- Limitation: toy ~19-instruction MIPS-style ISA, not full RV32I / floating point.

- Limitation: L1-only memory hierarchy; L2/L3 and main-memory wait states out of scope this phase.

- Limitation: in-order 5-stage pipeline; no out-of-order execution (Tomasulo/ROB) this phase.

- Future work: extend to full RV32I/RV64I; add multi-level cache and MESI/MOESI coherence; add out-of-order execution; compile the core to WebAssembly for client-side-only execution.

# References

[1] Hwang, I., Lee, J., Kang, H., Lee, G., & Kim, H. (2025). Survey of CPU and Memory Simulators in Computer Architecture. Simulation Modelling Practice and Theory, 138, 103032. https://doi.org/10.1016/j.simpat.2024.103032

[2] Mhatre, S. C., & Chandran, P. (2025). Simulators for Processors Used in Virtualization: A Survey. IEEE Open Journal of the Computer Society. https://doi.org/10.1109/OJCS.2025.3591359

[3] Mariotti, G., & Giorgi, R. (2022). WebRISC-V: A 32/64-bit RISC-V pipeline simulation tool. SoftwareX, 18, 101105. https://doi.org/10.1016/j.softx.2022.101105

[4] Große, D., Reitinger, S., & Klemmer, L. (2026). SonicRV: An Educational Platform for Web-Based Simulation and Visualization of RISC-V Processor Architectures. IEEE Access, 14, 13849-13864. https://doi.org/10.1109/ACCESS.2026.3656022

[5] Giorgi, R. (2025). FREESS: An Educational Simulator of a RISC-V-Inspired Superscalar Processor Based on Tomasulo's Algorithm. WCAE '25. https://doi.org/10.1145/3743646.3750018 (also arXiv:2506.07665)

[6] Muttillo, V., Valente, G., Federici, F., Pomante, L., Faccio, M., Tieri, C., & Ferri, S. (2016). A design methodology for soft-core platforms on FPGA with SMP Linux, OpenMP support, and distributed hardware profiling system. EURASIP Journal on Embedded Systems, 2016, 15. https://doi.org/10.1186/s13639-016-0051-9

[7] US Patent Application 2016/0063164 A1 — Method and System for Hardware/Software Co-simulation and Processor Resource Allocation.

[8] US Patent 7,788,078 B1 — Processor/Memory Co-Exploration at Multiple Abstraction Levels (CoWare Inc. / Synopsys Inc.).

[9] US Patent 9,928,179 B2 — Cache Replacement Policy (Intel Corporation).

[10] US Patent 6,263,427 B1 — Branch Prediction Mechanism (Rise Technology Co.).

[11] gem5 Project. https://www.gem5.org